COMPUTER SCIENCE Networks: IP Addressing & Transmission Media A 2.3.1 • A 2.3.2

| Name: | Class / Date: |
| --- | --- |
| Teacher: | Score / Grade: |

📚 Learning Objectives

A 2.3.1 — Describe different types of IP addressing (IPv4, IPv6, public/private, static/dynamic, NAT)

A 2.3.2 — Compare types of media for data transmission (fibre optic, twisted pair, wireless)

Activities are colour-coded by difficulty:

🌱 Foundation — Core 🔷 — Extension 🔥 — ⭐ Challenge

Instructions: Complete the activities at your level. Aim to attempt the next level when finished.

🌐 SECTION 1: IP ADDRESSING A 2.3.1 — Describe different types of IP addressing

📖 Key Knowledge — IP Addressing

• IP addresses are unique identifiers that allow devices to communicate over a network.

• IPv4: 32-bit address (~4.3 billion addresses) — e.g. 192.168.1.1

• IPv6: 128-bit address (~3.4 × 10³⁸ addresses) — e.g. 2001:0db8:85a3::8a2e:0370:7334

• Public IP: Globally unique, assigned by ISP, routable on the internet.

• Private IP: Used within a local network only, not directly accessible from outside.

• Static IP: Permanent — does not change. Ideal for servers and hosting.

• Dynamic IP: Assigned by DHCP, can change over time. Suitable for everyday devices.

• NAT (Network Address Translation): Allows multiple private IP devices to share one public IP.

🚀 STARTER ACTIVITY — IP Addressing

Work in the column that matches your level. You may move to the next when done.

| 🌱 Foundation | 🔷 Core | 🔥 Extension |
| --- | --- | --- |
| Q1: Think of a postal address. Write down ONE way it is similar to an IP address. | Q1: What is an IP address? What is its purpose? | Q1: Explain what 'IPv4 exhaustion' means and why it has happened. |
| Q2: Write down 3 devices in your home that connect to the internet. | Q2: IPv4 uses 32 bits. How many unique addresses does this create? Show your working. | Q2: Compare IPv4 and IPv6 — address format, number of addresses, and security. |
| Q3: True or False? Every device on the internet must have a unique IP address. | Q3: State ONE difference between a public and a private IP address. | Q3: Explain why private IPs are described as 'not routable' on the public internet. |
| Q4: What might happen if two devices share the same IP address? | Q4: Give ONE real-world example of when a static IP address is preferred over dynamic. | Q4: How does NAT solve the IPv4 exhaustion problem? Include an example. |

📝 ACTIVITY 1 — IP Addressing (Differentiated)

| 🌱 Foundation — Match & Define | 🔷 Core — Apply & Explain | 🔥 Extension — Evaluate & Analyse |
| --- | --- | --- |
| Q1: Match each term to its correct definition. Draw a line or write the letter. IPv4 • IPv6 • NAT • DHCP • Static IP • Dynamic IP | Q1: A school has 300 student laptops. Should they use static or dynamic IPs? Give TWO reasons. | Q1: A business runs a web server, 20 staff PCs and a CCTV system. Justify which devices need static vs dynamic IPs. |
| Q2: Fill in the gaps: A ________ IP address never changes and is often used by ________ . | Q2: Explain the difference between a public and private IP address using a real-life analogy. | Q2: Evaluate whether IPv6 completely eliminates the need for NAT. Support your view. |
| Q3: True or False? Private IP addresses can be directly accessed from the internet. | Q3: Describe TWO differences between IPv4 and IPv6. | Q3: Explain TWO security risks of static IP addresses and how each can be mitigated. |
| Q4: Name ONE advantage of a dynamic IP address over a static one. | Q4: Describe the steps of NAT when a home device visits a website. | Q4: Compare IPv4 and IPv6 across: address space, security, auto-config and compatibility. |

📊 Knowledge Check: Complete the IPv4 vs IPv6 Comparison Table

Fill in any missing cells from memory, then check against your notes.

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address Format | 32-bit |  |
| Address Count |  | 3.4 × 10³⁸ |
| Notation Style | Dotted decimal (e.g. 192.168.1.1) |  |
| Security |  | Integrated (IPSec built-in) |
| Auto-configuration | Limited |  |

⭐ Challenge 2A: Hospital Network Design

A hospital has: 200 networked medical devices, 500 staff computers, and 30 public patient information kiosks. Design an IP addressing scheme. For each group, state whether you would use: static or dynamic IPs, IPv4 or IPv6, and how NAT should be configured. Justify every decision.

⭐ Challenge 2B: Evaluate & Debate

"NAT is a temporary solution to a permanent problem." Discuss this statement. In your answer explain: what problem NAT solves, why it is considered a workaround, and whether the adoption of IPv6 makes NAT completely obsolete. Include at least ONE counter-argument.

⭐ Challenge 2C: Extended Writing (Exam Practice)

Compare IPv4 and IPv6 across FOUR criteria of your choice. For each criterion, explain the difference and evaluate which protocol is superior and why. Your answer should be structured, use technical terminology, and include at least one counter-argument or limitation.

📡 SECTION 2: TRANSMISSION MEDIA A 2.3.2 — Compare types of media for data transmission

📖 Key Knowledge — Transmission Media

• Fibre Optic: Transmits data as light pulses through glass/plastic. Very high bandwidth, low attenuation.

• Twisted Pair: Copper wires twisted together. Reduces EMI. Used in Ethernet/telephone networks.

• Wireless: Electromagnetic waves through the air. Includes Wi-Fi, cellular, satellite.

Key comparison factors:

Bandwidth — how much data can be sent per second

Attenuation — loss of signal strength over distance

Interference — disruption from external electromagnetic sources

Range — distance signal can travel without significant quality loss

Security — protection of data from interception

Cost — installation and running costs

Reliability — consistent delivery of data without errors

🚀 STARTER ACTIVITY — Transmission Media

| 🌱 Foundation | 🔷 Core | 🔥 Extension |
| --- | --- | --- |
| Q1: Name THREE ways you can connect a device to the internet. | Q1: Name the THREE types of transmission media covered in this topic. | Q1: Rank fibre optic, twisted pair and wireless by bandwidth. Justify your ranking. |
| Q2: What travels through a cable when you send data? Circle: Electricity / Light / Sound / Air | Q2: What is 'bandwidth' and why does it matter? | Q2: Explain how twisted pair cables reduce electromagnetic interference through their design. |
| Q3: Would you use Wi-Fi or cable in a hospital? Why? | Q3: Where would fibre optic be preferred over Wi-Fi? Give ONE reason. | Q3: Why is security a concern with wireless but less so with fibre optic? |
| Q4: Write ONE advantage and ONE disadvantage of wireless internet. | Q4: What is 'attenuation'? Which medium suffers from it most? | Q4: A company must connect two offices 50 km apart. Recommend a medium. Justify with THREE criteria. |

📝 ACTIVITY 1 — Transmission Media (Differentiated)

| 🌱 Foundation — Recall & Match | 🔷 Core — Apply & Explain | 🔥 Extension — Evaluate & Justify |
| --- | --- | --- |
| Q1: Match each medium to its description: Fibre Optic • Twisted Pair • Wireless | Q1: Explain why fibre optic cables have lower attenuation than twisted pair. | Q1: A new hospital needs internal networking. Compare fibre optic and twisted pair. Recommend one, justifying with bandwidth, security and cost. |
| Q2: Fill in: Fibre optic uses _______ pulses. Twisted pair uses _______ signals. | Q2: A school wants cheap internet in all classrooms. Recommend a medium. Give TWO reasons. | Q2: Analyse why wireless reliability is described as 'variable'. What factors cause this? |
| Q3: True or False? Twisted pair cables are immune to electromagnetic interference. | Q3: Describe how the twisted design of cables reduces interference. | Q3: Evaluate the trade-off between cost and performance when choosing transmission media for a large university campus. |
| Q4: Name ONE everyday device that uses wireless transmission. | Q4: Explain ONE reason why wireless transmission poses a security risk. | Q4: Construct an argument FOR or AGAINST using wireless-only transmission in a smart home. |

📊 Knowledge Check: Complete the Transmission Media Comparison Table

Fill in the blank cells from memory. Use: Very High / High / Moderate / Low / Variable / Immune.

| Feature | Fibre Optic | Twisted Pair | Wireless |
| --- | --- | --- | --- |
| Bandwidth |  | Moderate |  |
| Range | Very long |  | Variable |
| Interference Susceptibility | Immune |  |  |
| Attenuation |  | Moderate–High |  |
| Security | High |  | Variable |
| Cost (Installation) |  | Low | Variable |
| Reliability | Very high |  |  |

⭐ Challenge 2D: Smart City Infrastructure Design

A city is planning its digital infrastructure. It needs to connect: 50 schools, 200 street cameras, and 1000 smart traffic lights. For EACH system, recommend a transmission medium and justify your choice using at least THREE of the following: bandwidth, cost, security, range, reliability, interference.

⭐ Challenge 2E: Scenario Analysis

Analyse the following THREE environments for data transmission challenges. For each, identify the BIGGEST transmission challenge, explain which medium is most suitable, and analyse the trade-offs involved: (a) A factory floor with heavy machinery (b) An undersea connection between continents (c) A mobile emergency service vehicle

⭐ Challenge 2F: Evaluate & Debate (Exam Practice)

"As wireless technology improves, wired transmission will eventually become obsolete." To what extent do you agree with this statement? In your answer, consider: bandwidth, security, reliability, infrastructure cost, and physical constraints. Support your argument with specific examples and include at least ONE counter-argument.

✅ Reflection & Self-Assessment

| I can statement | 😊 Confident | 😐 Need to review |
| --- | --- | --- |
| I can describe what an IP address is and why it is needed. |  |  |
| I can explain the difference between IPv4 and IPv6. |  |  |
| I can distinguish between public and private IP addresses. |  |  |
| I can compare static and dynamic IP addresses with examples. |  |  |
| I can explain how NAT works and why it is used. |  |  |
| I can describe how fibre optic cables transmit data. |  |  |
| I can explain the role of the cladding in a fibre optic cable. |  |  |
| I can describe how twisted pair cables reduce interference. |  |  |
| I can explain how wireless transmission works. |  |  |
| I can compare transmission media using bandwidth, cost, range, security and reliability. |  |  |

🎟️ Exit Ticket

1.  Write down the MOST important thing you learned today.  [1 mark]

2.  Write down ONE question you still have.  [1 mark]

3.  Rate your confidence with both topics today (circle): 1  2  3  4  5