## 1. Describe two applications of GPUs other than graphics rendering. [3 marks]

Award 1 mark per valid application (max 2) and 1 mark for description.

- Machine learning / AI training – GPUs perform parallel matrix calculations efficiently, speeding up neural network training.

- Cryptocurrency mining – GPUs handle repetitive hash calculations simultaneously.

- Scientific simulations – Used in weather modelling, physics simulations, or bioinformatics due to parallel processing.

## 2. Describe how GPUs contribute to enhancing video game experiences. [3 marks]

Award marks for clear description.

- GPUs process large numbers of calculations in parallel, enabling real-time rendering.

- Support high frame rates and smooth animations, reducing lag.

- Enable advanced effects such as lighting, shadows, ray tracing, and physics simulations.

(1 mark per valid point, max 3)

## 3. Discuss how the design philosophy of CPUs and GPUs reflect their roles in computing systems. [5 marks]

Level-based marking

- 1–2 marks: Limited comparison, basic statements.

- 3–4 marks: Clear comparison with explanation.

- 5 marks: Balanced discussion, links design to function.

Indicative points:

- CPUs have few powerful cores, optimized for sequential and control-heavy tasks.

- GPUs have many smaller cores, optimized for parallel processing.

- CPUs focus on low latency, GPUs on high throughput.

- Reflects roles: CPU manages system operations, GPU accelerates data-parallel task

4. Describe the significance of energy efficiency in the design of CPUs and GPUs. [3 marks]

Award 1 mark per explained point.

- Reduces heat generation, improving reliability.

- Lowers power consumption, important for mobile and data centres.

- Enables higher performance per watt, improving sustainability.

5. Evaluate the effectiveness of the collaborative work between CPUs and GPUs in a computing system. [6 marks]

Level-based evaluation

- 1–2 marks: Simple description, no judgement.

- 3–4 marks: Some evaluation with advantages.

- 5–6 marks: Balanced evaluation with limitations.

Indicative points:

- CPU handles control, logic, and task scheduling.

- GPU accelerates parallel workloads.

- Improves overall system performance.

- Limitations include data transfer bottlenecks and programming complexity.

- Highly effective for tasks like gaming, AI, and simulations.

6. Describe the roles and characteristics of registers and cache memory within a computer system. [4 marks]

Award 2 marks per component.

Registers

- Smallest, fastest memory.

- Stores immediate data and instructions for execution.

Cache memory

- Stores frequently accessed data.

- Faster than RAM, slower than registers.

7a. Outline the role of ROM in a computer system. [2 marks]

- Stores firmware / boot instructions.

- Non-volatile; contents remain without power.

7b. Compare ROM with volatile memory types. [3 marks]

Award comparison points.

- ROM is non-volatile, RAM is volatile.

- ROM retains data after power loss; RAM does not.

- ROM stores permanent instructions; RAM stores temporary data.

## 8. Describe the primary functions of the data bus during the execute phase. [3 marks]

- Transfers data between CPU, memory, and I/O devices.

- Carries operands and results of execution.

- Supports bidirectional data flow.

9. Describe how the control bus facilitates interaction during a memory read operation. [3 marks]

- CPU sends a read signal via the control bus.

- Memory acknowledges the request.

- Synchronisation signals ensure correct timing.

10. Evaluate the impact of bus architecture on CPU performance. [4 marks]

Level-based

- 1–2 marks: Descriptive only.

- 3–4 marks: Evaluation with cause-effect.

Indicative points:

- Wider buses transfer more data simultaneously.

- Faster clock speeds improve throughput.

- Bottlenecks can limit CPU performance.

- Efficient bus design improves overall system speed.

11. Describe the basic principle of pipelining in multi-core architectures. [4 marks]

- Instruction execution divided into stages.

- Multiple instructions processed simultaneously.

- Each stage handled concurrently.

- Improves efficiency of instruction processing.

12. Describe how pipelining improves throughput. [3 marks]

- Multiple instructions completed per cycle.

- Reduces idle CPU stages.

- Increases instruction completion rate.

13. Describe challenges of pipelining in multi-core architectures. [3 marks]

- Data hazards (dependency conflicts).

- Control hazards (branching).

- Increased design complexity.

14. Evaluate the effectiveness of pipelining for highly parallel tasks. [6 marks]

Level-based evaluation

- 1–2 marks: Simple statements.

- 3–4 marks: Advantages explained.

- 5–6 marks: Balanced evaluation.

Indicative points:

- Highly effective due to parallel execution.

- Maximises CPU utilisation.

- Limited by synchronisation overhead and hazards.

- Most effective when tasks are independent.

15. Explain how data is stored and accessed on an optical disc. [6 marks]

Indicative points:

- Data stored as pits and lands.

- Laser reads reflected light differences.

- Spiral track from centre outward.

- Disc rotation speed varies.

- Error correction used.

- Read-only or rewritable depending on format.

16. Describe the advantages of NAS in a multi-device environment. [3 marks]

- Centralised data access.

- Enables file sharing across devices.

- Supports backups and redundancy.

17. Describe differences between internal HDDs and eMMCs. [3 marks]

- HDDs are mechanical; eMMCs are solid-state.

- HDDs offer larger capacity; eMMCs are faster and more durable.

- HDDs used in desktops; eMMCs in mobile devices.

18. Evaluate external SSDs for backup vs flash drives. [5 marks]

Level-based evaluation

Indicative points:

- SSDs are faster and more reliable.

- Better durability and lifespan.

- Flash drives are cheaper and portable.

- SSDs better for large, frequent backups.

- Cost is a limitation.

19. Evaluate the impact of NAND flash limitations on SSDs and eMMCs. [5 marks]

Indicative points:

- Limited write cycles affect longevity.

- Wear levelling mitigates degradation.

- Performance degrades over time.

- Storage density improves cost efficiency.

- Trade-off between speed, cost, and lifespan.